Steps for successful use of ES_PID_tune.m:

- Open g1_track.mdl

- Replace model (highlighted blue) with model of your choice

- Open ES_PID_tune.m

- Specify initial time and final time to create time vector (lines 11-13)

- Run simple ZN tests to obtain ZN PID parameters using Simulink model.  In the PID Controller subsystem set the gains after the integral and derivate control (labeled ‘zero for ZN tuning’) to zero, increase K until simulations yield a periodic output.  This provides you with Ku (ultimate gain) and Pu (period of oscillation): K = Ku*.6, Ti = Pu/2, Td = Pu/9

- Set gains after integral and derivate terms back to 1.

- In the ES_PID_tune m-file, enter your K, Ti, and Td from Ziegler Nichols tuning. (lines 18-20)

- Set a perturbation amplitude (alpha) for K, Ti, Td (ln 24-26) (note: alpha1, alpha2, alpha3 correspond to K, Ti, and Td respectively.  The index holds for adaptation gain as well)

- Set an adaptation gain (gamma) for K, Ti, Td (ln 30-32)

- Set modulation frequency (0<a<1) (ln 35).  These define your perturbation frequency omega s.t. ωi=π∗ai.  This satisfies the requirements in class that ω1≠ω2and ω1+ω2≠ω3.  For additional ideas for how to set your omegas, look at PID tuning via extremum seeking, pdf publication 80 on flyingv.ucsd.edu.

- Set high pass filter parameter (0<h<1) (ln 36)

- Set weighting for control effort for cost function (lambda) (ln 39).  For this simulation, cost function is J(θ(k))=1(Tf−to)∗e(t)2dt+λ∗1(Tf−0)u(t)2dt

Control effort weighting is set to zero for plants G1-G4.  Can use a cost function of your choosing, but this will need to be replaced in lines 63 and 84.

- Set time to start integrating error for cost function (t0) (ln 40)

- Set number of iterations for ES loop to run (N) (ln 42)

- Run the program.  Outputs will be evolution of cost function (J) and PID parameters